UNCLASSIFIED // FOR OFFICIAL USE ONLY

PRODUCT REQUIREMENTS DOCUMENT

3D Virtual Environment Simulation

Multi-Entity Autonomous Operations with UAV/Payload Integration

Version 1.1  |  2025-Q1  |  Aerospace & Defense Division

Supersedes: Version 1.0 (2025-Q1 Initial Baseline)

Target Implementation: Claude AI — Python 3.10+

# Document History & Revision Control

New requirements added in v1.1 are tagged [NEW]. Modified requirements are tagged [MOD]. All other content is unchanged from v1.0.

| Version | Date | Summary of Changes | Author |
| --- | --- | --- | --- |
| 1.0 | 2025-01-15 | Initial document — full baseline | PM — A&D Division |
| 1.1 | 2025-02-01 | M1 code review and fixes. New NFRs: NF-CE-001–005, NF-TE-001, NF-HA-001. New advanced-phase requirements: ADV-001–005. Updated NF-P-001 (reference CPU). New Sec. 16: M1 Code Review & Architecture. | PM — A&D Division |

# Table of Contents

# 1. Executive Summary

This Product Requirements Document (PRD) defines the full scope, architecture, functional and non-functional requirements, constraints, and acceptance criteria for the 3D Virtual Environment Simulation (3DVES) platform — a high-fidelity, Python-based simulation engine for operational prototyping, mission rehearsal, sensor algorithm validation, and autonomous system research in defence contexts.

Version 1.1 incorporates eleven new requirements arising from the M1 implementation review, a formal code audit of the submitted M1 codebase, architecture recommendations, and an advanced-phase feature set (combat, time-control, and swarm behaviour). All original v1.0 requirements remain in force.

# 2. Purpose & Scope

## 2.1 Purpose

The purpose of this document is to:

- Define the product vision and success criteria for the 3DVES platform.

- Establish unambiguous functional requirements decomposed by subsystem.

- Specify non-functional requirements including performance, scalability, code quality, and safety constraints.

- Provide sufficient design guidance for Claude AI to generate correct, tested, and maintainable Python code without further human clarification.

- Document the M1 code review findings and approved architecture.

## 2.2 In-Scope

- 3D simulation world with configurable terrain, structures, and airspace volumes.

- Entity model: pedestrians, wheeled vehicles, tracked vehicles, and UAVs.

- UAV autonomous flight management including waypoint navigation, geofencing, collision avoidance, and loiter rules.

- EO/IR optical payload simulation including FOV, gimbal articulation, detection range, and tracking quality metrics.

- Baseline matplotlib.pyplot real-time 2-D visualisation (M1), with incremental 3-D feature additions across milestones.

- Python API for scenario scripting and programmatic control.

- Simulation clock, event bus, replay, and logging infrastructure.

- Advanced phase: entity combat, rules of engagement, time-control modes, infinite runtime, and swarm behaviours.

## 2.3 Out of Scope

- Hardware-in-the-loop (HIL) integration with physical UAV autopilot systems.

- Classified sensor phenomenology models (ITAR-controlled performance data).

- Real-time network link / RF propagation simulation.

- Multiplayer / multi-operator collaborative control interfaces.

- Production deployment, CI/CD infrastructure, or cloud hosting.

# 3. Background & Operational Context

## 3.1 Problem Statement

Modern defence programmes require rapid, low-cost validation of autonomous sensor and platform behaviours before expensive field tests. Current simulation tools are either too high-fidelity (requiring classified hardware) or too abstract. There is a critical gap for a mid-fidelity, Python-native simulation environment that a small engineering team can configure, extend, and iterate within hours.

## 3.2 Operational Concept (CONOPS)

- A 3D world is initialised with configurable terrain and an urban/semi-urban entity population.

- Ground entities move per behavioural rules: pedestrians follow social force models; vehicles obey traffic-graph constraints and kinematic limits.

- One or more UAVs execute autonomous search routes over a defined Area of Interest (AOI).

- Each UAV carries one optical payload. The payload processes synthetic imagery within its FOV, running detection and classification algorithms.

- On EOI detection, the payload initiates persistent tracking at a defined quality level.

- Track data, metadata, and events are logged to structured JSONL for post-mission analysis.

# 4. Product Vision & Goals

## 4.1 Vision Statement

"To provide a self-contained, physics-grounded Python simulation that faithfully replicates the challenges of autonomous optical reconnaissance — enabling rapid algorithm prototyping, mission rehearsal, and quantitative performance benchmarking without classified hardware or field deployments."

## 4.2 Success Criteria

| I | D |
| --- | --- |
| SC-01 | All entity types operate concurrently at ≥ 30 simulation steps/sec with 200+ entities on the reference CPU (see NF-P-001). |
| SC-02 | UAV completes a configured search route while respecting all flight rules without human intervention. |
| SC-03 | Optical payload achieves ≥ 85% EOI detection rate under nominal scenario conditions. |
| SC-04 | Payload maintains track quality ≥ MEDIUM on acquired targets for ≥ 90% of engagement duration. |
| SC-05 | Full scenario replay is bit-exact and reproduces identical event sequences. |
| SC-06 | Python API enables a new scenario to be scripted and executed within 30 minutes by a trained user. |
| SC-07 | All requirements in this PRD have at least one corresponding automated test. |
| SC-08 | All delivered Python code passes PEP8 linting (flake8 zero violations) and mypy type checking (zero errors). |

# 5. User Stories & Use Cases

## 5.1 User Stories

- As a Mission Engineer, I want to define a scenario via a YAML/JSON config so that I can configure terrain, entity populations, UAV routes, and payload parameters without modifying code.

- As a Mission Engineer, I want real-time matplotlib visualisation from M1 onwards so that I can observe simulation state during development without installing separate tools.

- As an Algorithm Developer, I want to inject a custom detection algorithm via a Python plugin interface so that I can benchmark my algorithm against the baseline.

- As an Algorithm Developer, I want per-entity ground-truth visibility flags at each timestep so that I can compute precision/recall for my detector.

- As a Systems Engineer, I want the UAV subsystem to enforce configurable no-fly zones so that I can validate geofence logic.

- As a Systems Engineer, I want payload FOV and tracking quality metrics exported per-step so that I can verify tracking requirement compliance.

## 5.2 Key Use Cases

| U | s |
| --- | --- |
| UC-01: Area Search | UAV executes lawnmower search pattern over AOI; payload logs all entity sightings. |
| UC-02: EOI Track Acquisition | Payload detects EOI; transitions to cued search then persistent tracking mode. |
| UC-03: Track Handoff | Primary UAV approaches fuel limit; hands off active track to a secondary UAV. |
| UC-04: EOI Evasion | EOI moves behind occluder; payload must re-detect after LOS restoration. |
| UC-05: Geofence Violation | UAV approaches boundary; flight controller initiates safe orbit or RTB. |
| UC-06: Multi-UAV Deconfliction | Two UAVs converge on same EOI; deconfliction assigns PRIMARY/SECONDARY roles. |
| UC-07: Algorithm Benchmarking | External detection algorithm runs in parallel with baseline; metrics compared. |
| UC-08: Squad Movement (Advanced) | Pedestrian squad navigates as a cohesive unit with leader-follower dynamics. |
| UC-09: Rules of Engagement (Advanced) | Armed entity engages only targets satisfying ROE criteria within weapon range. |
| UC-10: Accelerated Playback (Advanced) | Operator runs scenario at 10× wall-clock speed; visualiser renders key-frame every N steps. |

# 6. Functional Requirements

## 6.1 Simulation World & Environment

### 6.1.1 Terrain & Spatial Model

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| ENV-001 | Terrain | The system SHALL represent the world as a 3D Cartesian coordinate space with configurable X/Y extent (metres) and Z elevation derived from a DEM or flat-plane configuration. | MUST |
| ENV-002 | Coordinate System | The system SHALL use ENU (East-North-Up) convention internally. WGS-84 lat/lon import/export SHALL be supported. | MUST |
| ENV-003 | Structures | The system SHALL support placement of AABB structures that occlude line-of-sight calculations. | MUST |
| ENV-004 | Vegetation | The system SHALL support configurable vegetation density regions that degrade optical detection probability. | SHOULD |
| ENV-005 | Airspace Volumes | The system SHALL support cylindrical and rectangular restricted airspace volumes (No-Fly Zones). | MUST |
| ENV-006 | Road Network | The system SHALL support a directed graph road network for vehicle navigation. | MUST |
| ENV-007 | Scenario Config | All world parameters SHALL be configurable via external YAML or JSON without code modification. | MUST |

### 6.1.2 Simulation Clock & Event System

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| SIM-001 | Discrete Timestep | The simulation SHALL advance via a fixed discrete timestep (dt) configurable from 0.05 s to 1.0 s. | MUST |
| SIM-002 | Event Bus | The system SHALL provide a publish-subscribe event bus for decoupled inter-subsystem communication. | MUST |
| SIM-003 | Determinism | Given identical seed and configuration, the simulation SHALL produce bit-exact identical results across runs. | MUST |
| SIM-004 | Replay | The simulation SHALL log full entity state at each step to enable deterministic replay. | MUST |
| SIM-005 | Headless Mode | The simulation SHALL execute without a display, controlled entirely via API. | MUST |
| SIM-006 | Real-time Mode | The simulation SHALL optionally pace to wall-clock time for visualisation (see NF-VIZ-001). | SHOULD |

## 6.2 Entity Subsystem

### 6.2.1 Entity Base Model

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| ENT-001 | Base Entity | All entities SHALL share a common abstract base class with: unique ID, EntityType enum, 3-D ENU position, heading (degrees), velocity vector, alive flag, FSM state, EOI flag, and signature factor. | MUST |
| ENT-002 | Entity Types | The system SHALL support EntityType values: PEDESTRIAN, WHEELED_VEHICLE, TRACKED_VEHICLE, UAV — defined as a Python Enum (not raw strings). | MUST |
| ENT-003 | State Machine | Each entity SHALL implement a configurable FSM governing behavioural modes, represented as a Python Enum. | MUST |
| ENT-004 | Plugin Interface | Entity behaviour SHALL be overridable via a Python abstract base class without modifying core entity code. | MUST |
| ENT-005 | Collision Model | Ground entities SHALL use circular footprint collision detection to prevent overlap. | SHOULD |

### 6.2.2 Pedestrian Entity Rules

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| PED-001 | Kinematics | Pedestrians SHALL move within [PED_SPEED_MIN, PED_SPEED_MAX] (default: 1.0–1.8 m/s). Speed MUST be normalised to target speed; additive Cartesian noise that grows speed unboundedly is NOT permitted. | MUST |
| PED-002 | Waypoint Nav | Pedestrians SHALL navigate to waypoints using straight-line paths, replanning when blocked. | MUST |
| PED-003 | Social Force | Pedestrians SHALL apply a simplified social force model to avoid mutual collisions. | SHOULD |
| PED-004 | EOI Flag | Pedestrians MAY be flagged as Entity of Interest (EOI) for targeting by payloads. | MUST |
| PED-005 | Concealment | Pedestrians SHALL model partial concealment when within a vegetation region. | SHOULD |

### 6.2.3 Terrain Adherence

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| PED-006 [NEW] | Terrain Lock | Pedestrian Z position SHALL always equal the terrain elevation at (x, y). Z velocity SHALL always be 0.0. Random Z values from numpy generators SHALL NOT be used as pedestrian altitude. | MUST |
| VEH-007 [NEW] | Vehicle Terrain Lock | Vehicle Z position SHALL always equal the terrain elevation at (x, y). Off-road vehicles follow terrain slope. | MUST |

### 6.2.4 Vehicle Entity Rules

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| VEH-001 | Road Constraint | Wheeled vehicles SHALL follow the road network graph; tracked vehicles MAY traverse off-road at reduced speed. | MUST |
| VEH-002 | Kinematics | Vehicles SHALL be modelled with configurable max speed, acceleration, deceleration, and turning radius. | MUST |
| VEH-003 | Pathfinding | Vehicles SHALL use A* pathfinding on the road network graph. | MUST |
| VEH-004 | Speed Zones | Vehicles SHALL respect configurable speed-limit zones in the world. | SHOULD |
| VEH-005 | EOI Flag | Vehicles MAY be designated as EOI. | MUST |
| VEH-006 | Signature | Each vehicle SHALL carry a configurable thermal/visual signature parameter in [0,1]. | MUST |

## 6.3 UAV Subsystem

### 6.3.1 UAV Platform Model

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| UAV-001 | 3D Flight | The UAV SHALL operate in full 3D space with configurable altitude envelope (min/max AGL). | MUST |
| UAV-002 | Kinematics | The UAV SHALL model speed (m/s), climb/descent rate, bank angle limit, and turn rate. | MUST |
| UAV-003 | Fuel/Endurance | The UAV SHALL model an endurance budget (flight-time seconds) and trigger low-fuel alerts. | MUST |
| UAV-004 | Multi-UAV | The simulation SHALL support simultaneous operation of up to N UAVs (configurable, default N=4). | MUST |
| UAV-005 | Autopilot Modes | The UAV SHALL support autopilot modes: WAYPOINT, LOITER, ORBIT, RTB, EMERGENCY_LAND — all as a Python Enum. | MUST |

### 6.3.2 UAV Flight Rules

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| FLR-001 | NFZ Avoidance | The UAV SHALL detect upcoming NFZ penetration ≥ 10 s ahead and execute lateral avoidance. | MUST |
| FLR-002 | Altitude Floor | The UAV SHALL not descend below configurable minimum AGL (default: 30 m). | MUST |
| FLR-003 | Altitude Ceiling | The UAV SHALL not ascend above configurable maximum AGL (default: 500 m). | MUST |
| FLR-004 | Collision Avoidance | The UAV SHALL maintain ≥ 50 m separation from all other airborne entities. | MUST |
| FLR-005 | Geofence | The UAV SHALL not exit the scenario boundary. On approach, SHALL orbit or RTB. | MUST |
| FLR-006 | Low Fuel RTB | On reaching low-fuel threshold, UAV SHALL complete current task waypoint then RTB. | MUST |
| FLR-007 | Wind Model | The UAV SHALL be subject to a configurable wind vector adding drift to position. | SHOULD |
| FLR-008 | Search Pattern | The UAV SHALL support lawnmower, expanding-spiral, and random-walk search patterns. | MUST |
| FLR-009 | Cued Slew | On receiving a cue from the payload, the UAV SHALL orbit the cue point at configurable radius/altitude. | MUST |
| FLR-010 | Deconfliction | When multiple UAVs are assigned the same EOI, a priority algorithm SHALL assign PRIMARY/SECONDARY roles. | MUST |

## 6.4 Optical Payload Subsystem

Requirements PAY-001 to PAY-007, POL-001 to POL-011, and LOG-001 to LOG-007 are unchanged from PRD v1.0 §6.4–6.6 and remain in force. Refer to v1.0 for full text.

# 7. Visualisation Requirements

This section is new in v1.1, consolidating visualisation requirements raised during the M1 review.

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-VIZ-001 [NEW] | M1 Baseline Viz | From M1, the simulation SHALL ship a real-time 2-D top-down matplotlib.pyplot visualiser executed in the same process as the simulation loop. | MUST |
| NF-VIZ-002 [NEW] | Entity Differentiation | The visualiser SHALL render each EntityType in a distinct colour. EOI entities SHALL have a visually distinct marker (e.g., red edge ring). | MUST |
| NF-VIZ-003 [NEW] | Heading Arrows | The visualiser SHALL render a velocity-direction arrow for each living entity. | MUST |
| NF-VIZ-004 [NEW] | Sim Time Display | The visualiser title SHALL display current simulation time, step index, alive count, and dead count. | MUST |
| NF-VIZ-005 [NEW] | Legend | The visualiser SHALL include a legend mapping entity type to colour and EOI marker. | MUST |
| NF-VIZ-006 [NEW] | Incremental Features | Each subsequent milestone release SHALL add visualisation of its new features (e.g., M3: NFZ circles; M4: FOV cone; M5: track ellipses; M6: road graph overlay). | MUST |
| NF-VIZ-007 [NEW] | GPU Acceleration | Where GPU acceleration is enabled (NF-HA-001), visualisation batching SHALL use CuPy or equivalent to vectorise position array assembly before passing to matplotlib. | SHOULD |

# 8. Non-Functional Requirements

## 8.1 Performance

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-P-001 [MOD] | Throughput | The simulation SHALL sustain ≥ 30 steps/second with ≤ 200 entities on the reference system: 12th Gen Intel Core i5-12450H (2.00 GHz base, 8 cores / 12 threads), 16 GB DDR4 RAM. Previously referenced as 'Intel i7, 16 GB RAM'. | MUST |
| NF-P-002 | Scalability | Performance SHALL degrade gracefully. With 500 entities, throughput SHALL remain ≥ 10 steps/second on the reference system. | MUST |
| NF-P-003 | Memory | Peak memory usage SHALL not exceed 4 GB for a 200-entity, 60-minute scenario. | MUST |
| NF-P-004 | LOS Raycast | LOS raycasts SHALL complete in ≤ 2 ms per query (amortised over batch). | MUST |

## 8.2 Reliability & Correctness

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-R-001 | Determinism | Identical seed + config SHALL produce identical state sequences across platforms (SIM-003). | MUST |
| NF-R-002 | Physics Stability | No entity SHALL exhibit unbounded velocity or position drift over a 1-hour simulation. | MUST |
| NF-R-003 | Error Handling | All configuration errors SHALL raise descriptive exceptions before simulation start. | MUST |

## 8.3 Code Engineering & Quality (New in v1.1)

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-CE-001 [NEW] | PEP8 Compliance | All Python source files SHALL comply with PEP8. flake8 SHALL report zero violations at each release. Line length limit: 99 characters. | MUST |
| NF-CE-002 [NEW] | Type Annotations | All function and method signatures SHALL include complete Python type annotations. mypy SHALL report zero errors in strict mode at each release. | MUST |
| NF-CE-003 [NEW] | Docstrings | All public classes, methods, and functions SHALL have NumPy-format docstrings including Parameters, Returns, and Raises sections where applicable. | MUST |
| NF-CE-004 [NEW] | Design Patterns | Design patterns SHALL be applied where they demonstrably improve maintainability. Mandatory patterns for M1–M3: Facade (SimulationEngine), Observer (EventBus), Template Method (Entity.step()), Registry (EntityManager), Context Manager (Logger). Additional patterns SHALL be documented in the relevant module docstring. | MUST |
| NF-CE-005 [NEW] | Inline Comments | Non-trivial algorithmic logic, physics constraints, and all PRD requirement references SHALL be annotated with inline comments citing the relevant Req ID (e.g., # PED-001, # Req-7). | MUST |

## 8.4 Testing (New in v1.1)

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-TE-001 [NEW] | Unit Tests at Every Release | Every code release SHALL include unit tests covering all new and modified modules. A release without tests is NOT acceptable, regardless of milestone scope. Test coverage SHALL be ≥ 80% for all core modules. | MUST |
| NF-TE-002 [NEW] | Requirement Traceability | Every test function docstring SHALL cite the Req ID(s) it validates. | MUST |
| NF-TE-003 [NEW] | Regression Guard | Existing passing tests SHALL not be removed or suppressed without PM written approval. | MUST |
| NF-TE-004 [NEW] | Integration Test | Each milestone release SHALL include at least one end-to-end integration test that runs the full engine headlessly for a minimum of 100 steps and validates JSONL output. | MUST |

## 8.5 GPU Acceleration (New in v1.1)

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-HA-001 [NEW] | GPU Utilisation | Where a CUDA-capable GPU is present, the simulation SHALL utilise GPU acceleration via CuPy for: batch entity position updates (EntityManager.step_all), LOS raycast batches (DetectionEngine), and Kalman filter matrix operations (TrackManager). GPU acceleration SHALL be optional — the system MUST fall back gracefully to NumPy on CPU-only machines. | SHOULD |
| NF-HA-002 [NEW] | GPU Abstraction | A GPU/CPU abstraction layer (e.g., xp = cupy if GPU else numpy) SHALL be used throughout so that compute kernels need not be written twice. | SHOULD |

## 8.6 Maintainability & Extensibility

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-M-001 | Architecture | The codebase SHALL use a modular component architecture. Each subsystem SHALL be independently testable. | MUST |
| NF-M-002 | Plugin Interface | Entity behaviour, detection algorithms, and payload models SHALL be overridable via Python abstract base classes. | MUST |
| NF-M-003 | Test Coverage | Unit test coverage SHALL be ≥ 80% for all core modules. | MUST |
| NF-M-004 | Documentation | All public Python classes and methods SHALL have NumPy-format docstrings (see NF-CE-003). | MUST |
| NF-M-005 | Type Annotations | All function signatures SHALL include Python type annotations (see NF-CE-002). | MUST |
| NF-M-006 | Configuration | All tunable parameters SHALL be externalized to a SimDefaults dataclass — no magic numbers in code. | MUST |

## 8.7 Portability

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NF-PT-001 | Python Version | The system SHALL be compatible with Python 3.10 and above. | MUST |
| NF-PT-002 | Platform | The system SHALL run on Windows 10+, macOS 12+, and Ubuntu 22.04+ without OS-specific code paths. | MUST |
| NF-PT-003 | Dependencies | All dependencies SHALL be installable via pip from PyPI. No private package registry required. | MUST |

# 9. Advanced Phase Requirements (New in v1.1)

The following requirements define the post-M6 advanced phase. They SHALL NOT block delivery of M1–M6 milestones. Each will be decomposed into a sub-milestone at the start of the advanced phase.

## 9.1 Combat & Rules of Engagement

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| ADV-001 [NEW] | Entity Weapons | Armed entities (PEDESTRIAN and VEHICLE subtypes) SHALL carry configurable weapon definitions: weapon type, range (m), accuracy (0–1), damage (0–100), rate of fire (rounds/s), and ammunition count. | SHOULD |
| ADV-002 [NEW] | Rules of Engagement | Each armed entity SHALL have an ROE profile defining: engagement authorisation criteria (EOI only / all hostile / self-defence), minimum identification confidence threshold, and collateral damage avoidance radius. | SHOULD |
| ADV-003 [NEW] | Weapon Physics | Weapon projectiles SHALL apply ballistic physics (range drop-off, accuracy degradation with range) consistent with the weapon definition stats. Lethal hits SHALL call entity.kill(). | SHOULD |

## 9.2 Time Control Modes

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| ADV-004 [NEW] | Accelerated Time | The simulation SHALL support a configurable time-acceleration factor X (default: 1.0, max: 100.0). At X > 1, the visualiser SHALL render every ⌈X⌉-th step only to maintain interactive frame rate. All logging SHALL remain at full-fidelity (every step). | SHOULD |
| ADV-005 [NEW] | Infinite Runtime | The simulation SHALL support an 'infinite' duration mode where the scenario runs until explicitly stopped via API or KeyboardInterrupt, without pre-computing a step count. | SHOULD |

## 9.3 Swarm & Squad Behaviour

| Req ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| ADV-006 [NEW] | Pedestrian Squads | Pedestrians SHALL be groupable into squads. Each squad SHALL have a designated leader entity. Squad members SHALL apply Reynolds' flocking rules (cohesion, separation, alignment) relative to the leader's position and heading. | SHOULD |
| ADV-007 [NEW] | Squad Tactics | Squads SHALL support tactical formation presets: LINE, WEDGE, COLUMN. Formation geometry SHALL update continuously based on leader heading. | COULD |

# 10. System Architecture

## 10.1 Module Structure

The approved Python package structure is:

sim3dves/

├── config/          # SimDefaults — all scalar constants (NF-M-006)

├── core/            # SimulationEngine (Facade), World, EventBus, SimulationConfig

├── entities/        # Entity (ABC), EntityManager (Registry), Pedestrian, Vehicle, UAV

├── payload/         # OpticalPayload, DetectionEngine, TrackManager

├── world/           # Terrain, AABB, NFZCylinder, RoadNetwork

├── logging/         # Logger (Context Manager), ReplayWriter, DataExporter

├── viz/             # DebugPlot (matplotlib), future 3-D renderer

├── config/          # ScenarioLoader, ConfigValidator

└── tests/           # unittest suite mirroring package structure (NF-TE-001)

## 10.2 Design Pattern Summary

| P | a |
| --- | --- |
| Facade | SimulationEngine — single entry point hiding EntityManager, EventBus, Logger, World complexity. |
| Observer | EventBus — decouples publishers (engine, entities) from subscribers (logger, payload, UAV). |
| Template Method | Entity.step() — fixed algorithm skeleton; _update_behavior() and _update_kinematics() are abstract hooks. |
| Registry | EntityManager — authoritative, indexed collection with living(), by_type(), and batch step_all() interfaces. |
| Context Manager | Logger — guarantees JSONL file flush/close even on exception mid-simulation. |
| Strategy (M2+) | Behaviour plugins — injectable detection algorithm and payload models via ABC. |
| State (M2+) | Autopilot FSM — UAV mode transitions (WAYPOINT → ORBIT → RTB) as explicit State objects. |

# 11. Configuration Schema

All scenario parameters SHALL be expressed in a single root YAML file. See PRD v1.0 §9 for the normative schema. The schema is unchanged in v1.1 except for the addition of the viz block:

viz:
  enabled: true            # Enable/disable real-time visualiser
  render_every_n_steps: 1  # Set > 1 for accelerated-time mode (ADV-004)
  show_dead: false         # Include dead entities in render
  window_title: "3DVES Debug View"
advanced:                  # Advanced phase (ADV-001 to ADV-007)
  time_acceleration: 1.0   # X factor (1.0 = real-time)
  infinite_mode: false     # If true, ignore duration_s
  squads: []               # List of {leader_id, member_ids, formation}

# 12. Acceptance Criteria & Test Plan

## 12.1 M1 Acceptance Test Cases

| T | e |
| --- | --- |
| ATC-M1-001: Terrain Lock | Spawn 50 pedestrians with random initial Z. After 100 steps, ALL pedestrian Z positions equal terrain elevation. PASS: zero terrain violations. |
| ATC-M1-002: Speed Envelope | Run 50 pedestrians for 500 steps. PASS: XY speed of every living pedestrian ∈ [1.0, 1.8] m/s at every measured step. |
| ATC-M1-003: Boundary Kill | Entity placed 1 step from boundary exits and is killed within 2 steps. PASS: alive=False confirmed. |
| ATC-M1-004: Determinism | Run identical scenario twice with seed=42. PASS: final position lists are byte-equal. |
| ATC-M1-005: Dead Entity Frozen | Kill entity; run 10 more steps. PASS: position unchanged. |
| ATC-M1-006: Logger Integrity | Run 10-step scenario. PASS: JSONL has 10 step records; every record contains wall_dt_ms and entity_count. |
| ATC-M1-007: Logger Closed | Introduce exception mid-run inside 'with Logger'. PASS: file.closed is True after exception. |
| ATC-M1-008: Duplicate ID Rejected | Add entity with duplicate ID. PASS: ValueError raised immediately. |
| ATC-M1-009: Event Bus | Subscribe handler to OUT_OF_BOUNDS; trigger boundary exit. PASS: handler called exactly once with correct event_type. |
| ATC-M1-010: PEP8 | Run flake8 on all M1 source files. PASS: zero violations. |

# 13. Risks & Mitigations

| Risk ID | Description | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| RSK-001 | Physics instability at high dt values causes entity position explosion. | Medium | High | Enforce dt ≤ 1.0 s; add velocity clamps; unit test for divergence. |
| RSK-002 | LOS raycast becomes performance bottleneck with many entities. | High | High | Batch raycasts using NumPy BVH tree; limit raycasts to FOV subset. GPU acceleration (NF-HA-001) for M4+. |
| RSK-003 | Kalman filter diverges on highly maneuvering EOI. | Medium | Medium | Tune process noise; implement IMM as fallback (M5+). |
| RSK-004 | Road A* pathfinding fails on disconnected road graph. | Medium | Medium | Validate graph connectivity at load; raise descriptive config error. |
| RSK-005 | Multi-UAV deconfliction introduces priority deadlocks. | Low | High | Time-based priority token with TTL; unit test deadlock scenario. |
| RSK-006 | GPU acceleration unavailable on target hardware — sim falls back to CPU. | Low | Medium | NF-HA-001 explicitly requires graceful CPU fallback; xp abstraction layer guards this. |
| RSK-007 | Advanced phase ROE logic introduces emergent simulation instability. | Medium | High | Implement ROE as isolated strategy; comprehensive integration tests before enabling. |

# 14. Roadmap & Milestones

| Milestone | Deliverable | Target | Owner |
| --- | --- | --- | --- |
| M1 | SimulationEngine, World (Terrain, AABB, NFZ), EventBus, EntityManager, PedestrianEntity (terrain lock, speed envelope), Logger (context manager), DebugPlot (matplotlib real-time 2-D viz), full M1 unit tests. | Week 2 | Claude |
| M2 | VehicleEntity (A* road nav, kinematic model, terrain lock), full entity FSM, social force pedestrian update. | Week 4 | Claude |
| M3 | UAVEntity (3-D kinematics, autopilot FSM: WAYPOINT/LOITER/ORBIT/RTB). All flight rules (FLR-001–010). NFZ circles in viz. | Week 6 | Claude |
| M4 | OpticalPayload, DetectionEngine (P(D) model, LOS raycast, gimbal FSM). FOV cone in viz. | Week 8 | Claude |
| M5 | TrackManager (Kalman filter, track quality metrics, re-acquisition). Track ellipses in viz. | Week 10 | Claude |
| M6 | Multi-UAV deconfliction, track handoff protocol, ScenarioLoader (YAML), ReplayWriter, DataExporter. Road graph in viz. | Week 12 | Claude |
| M7 | Full acceptance test suite (ATC-001–010). Performance tuning (NF-P-001/002). GPU acceleration (NF-HA-001). | Week 13 | Claude / QA |
| M8 | Advanced Phase: ADV-001–003 (weapons, ROE, ballistics). | Week 16 | Claude |
| M9 | Advanced Phase: ADV-004–005 (accelerated time, infinite mode). | Week 17 | Claude |
| M10 | Advanced Phase: ADV-006–007 (squad / swarm behaviour). | Week 18 | Claude |
| M11 | Documentation, code review, PRD delta review, baseline release. | Week 19 | PM + Claude |

# 15. Glossary

| T | e |
| --- | --- |
| 3DVES | 3D Virtual Environment Simulation — the system described in this PRD. |
| AABB | Axis-Aligned Bounding Box — rectangular structure primitive used for occlusion. |
| AOI | Area of Interest — geospatial polygon defining UAV search area. |
| AGL | Above Ground Level — altitude referenced to terrain elevation. |
| DEM | Digital Elevation Model — raster height data for terrain surface. |
| ENU | East-North-Up — local Cartesian coordinate convention used internally. |
| EOI | Entity of Interest — entity designated as payload tracking target. |
| EO/IR | Electro-Optical / Infrared — optical payload sensor modalities. |
| FOV | Field of View — angular cone subtended by the payload sensor. |
| FSM | Finite State Machine — behavioural control architecture. |
| GPU | Graphics Processing Unit — used for parallel compute acceleration (NF-HA-001). |
| IMM | Interacting Multiple Model — adaptive Kalman filter for maneuvering targets. |
| LOS | Line of Sight — unobstructed optical path between sensor and target. |
| NFZ | No-Fly Zone — restricted airspace volume. |
| P(D) | Probability of Detection — per-step likelihood of detecting an entity in FOV. |
| ROE | Rules of Engagement — criteria governing when and how armed entities may engage. |
| RTB | Return to Base — autopilot mode directing UAV to launch point. |
| UAV | Unmanned Aerial Vehicle — autonomous airborne platform. |
| WGS-84 | World Geodetic System 1984 — global geodetic reference. |

# 16. M1 Code Review — Findings & Architecture Recommendations

This section documents the formal review of the M1 code submission against PRD v1.0 requirements and v1.1 NF-CE, NF-TE requirements. All findings have been corrected in the M1 Revised Baseline delivered alongside PRD v1.1.

## 16.1 Issue Register

Severity levels: CRITICAL = correctness violation of a MUST requirement; HIGH = design deficiency causing downstream technical debt; LOW = code quality / style.

| File | Area | Severity | Issue | Fix Applied |
| --- | --- | --- | --- | --- |
| pedestrian.py | velocity update | CRITICAL | velocity += randn(3)*0.1 adds Z component — violates Req-7 (terrain lock) and PED-001 (unbounded speed growth). | Replaced with angular rotation of XY velocity vector preserving magnitude; Z velocity zeroed on every step. |
| base.py | Entity dataclass | HIGH | @dataclass with mutable numpy array fields — causes hashing, equality, and __repr__ bugs. entity_type: str uses raw string (violates ENT-002). | Replaced dataclass with explicit __init__. EntityType and EntityState defined as Python Enums. kill() helper added for clean FSM transition. |
| base.py | EntityManager | HIGH | add() silently overwrites duplicate entity_id — masks scenario config errors. No living() method — dead entities passed to logger. No by_type() — needed by payload FOV query. | Duplicate ID raises ValueError. living(), by_type(), remove(), __len__, __iter__ added. |
| event_bus.py | Event.type | HIGH | type: str uses raw strings — typo-prone, no IDE completion. Comment claimed thread-safety where none exists. | EventType enum introduced. Misleading thread-safety comment removed. unsubscribe() added. |
| engine.py | RNG seeding | HIGH | np.random.seed() (deprecated global RNG) used. logger.close() called after the step loop — file handle leaks on exception. | np.random.default_rng(seed) used as primary RNG. Logger now used as context manager inside run(). |
| engine.py | entity iteration | HIGH | Two full entity iterations per step (step_all + separate bounds loop) — O(2n) instead of O(n). | Merged into single pass over living entities; dead entities short-circuit inside Entity.step(). |
| engine.py | event logging | HIGH | EventBus subscribed but logger never wired to it — OUT_OF_BOUNDS events not reaching JSONL (violates LOG-002). | Engine subscribes _handle_out_of_bounds() to EventBus in __init__; events reach JSONL automatically. |
| logger.py | file handle | HIGH | File opened in __init__; close() called manually after run() — handle leaks on any exception. filepath: str not Path. | Context manager protocol (__enter__/__exit__) implemented. filepath type changed to pathlib.Path. |
| logger.py | log record | LOW | Missing fields: heading_deg, alive, is_eoi, state, wall_dt_ms. entity_type logged as raw string. | All fields added. entity_type logged via .name. wall_dt_ms performance column added (LOG-005). |
| world.py | World dataclass | HIGH | @dataclass holding bare numpy array — equality issues. No Terrain model, no snap_to_terrain(), no NFZ query, no structure query. | Replaced with full World class exposing snap_to_terrain(), in_nfz(), occluded_by_structure(). Terrain, AABB, NFZCylinder primitives added. |
| debug_plot.py | visualiser | HIGH | No entity-type colour, no heading arrows, no EOI indicator, no dead-entity filter, no sim-time title, no legend (violates NF-VIZ-001 to NF-VIZ-005). | Full DebugPlot rewrite: type colours, heading arrows, EOI ring, dead entity filter, sim-time title, legend. |
| run_simulation.py | spawning | CRITICAL | np.random.rand(3)*max_x gives random Z — violates Req-7. | Replaced with World.snap_to_terrain() call. |
| run_simulation.py | imports | HIGH | from sim3dves.viz.debug_plot import DebugPlot — viz/__init__.py missing → ImportError at runtime. | All package __init__.py files created. |
| run_simulation.py | logger | HIGH | sim.step() loop outside logger context manager — file handle leaks on KeyboardInterrupt. | Step loop wrapped in with sim._logger. |
| (all files) | magic numbers | LOW | Scalar constants (0.1, 50.0, max_x, 1.8 etc.) hardcoded inline — violates NF-M-006. | SimDefaults frozen dataclass introduced; all scalars reference it. |
| (all files) | type hints | LOW | Several methods lack full type annotations — violates NF-CE-002. | All signatures annotated throughout revised baseline. |
| test_smoke.py | OOB test | LOW | test_out_of_bounds_kills_entity used velocity=10 m/s but pedestrian normalises speed to ≤1.8 m/s — entity never reached boundary in test duration. | Test corrected: entity placed at X=99.95 m with speed_mps=PED_SPEED_MAX_MPS to guarantee exit in 1 step. |

## 16.2 Architecture Comparison

| Component | Original (M1 Submission) | Revised (M1 Baseline) |
| --- | --- | --- |
| Entity base | @dataclass with mutable numpy fields; entity_type: str. | Abstract class (ABC) with explicit __init__; EntityType/EntityState Enums; Template Method step() skeleton with abstract hooks. |
| EntityManager | Dict with no lifecycle guards. | Registry pattern: add() raises on dup; living(), by_type(), remove(), __len__, __iter__ added. |
| EventBus | Raw string event types; unsubscribe() missing; misleading thread-safety claim. | EventType Enum; subscribe/unsubscribe/publish; no false thread-safety claim. |
| World | @dataclass holding bare np.ndarray; XY-only in_bounds(). | Facade class: Terrain, AABB, NFZCylinder sub-models; snap_to_terrain(); in_nfz(); occluded_by_structure(). |
| Logger | Open in __init__, manual close(); str filepath. | Context Manager protocol; Path filepath; log_step + log_event + wall_dt_ms. |
| SimulationEngine | Deprecated np.random.seed(); two-pass entity loop; logger not context-managed; events not wired to log. | np.random.default_rng(); single-pass living loop; Logger as context manager; event handler wired. |
| PedestrianEntity | Additive randn(3) noise → unbounded speed, Z violation. | Angular rotation preserving speed magnitude; terrain lock enforced every step. |
| DebugPlot | Single colour, no arrows, no legend, no sim-time title. | Type colours, heading arrows, EOI ring, legend, sim-time title, dead-entity handling. |
| SimDefaults | None — all scalars hardcoded inline. | Frozen SimDefaults dataclass: single source of truth for all constants. |
| Package init files | Missing viz/__init__.py → ImportError. | All __init__.py created for complete importable package. |

## 16.3 M1 Test Results (28 Tests)

| Test ID | Description | Result | Req Coverage |
| --- | --- | --- | --- |
| T-ENT-001 | EntityType and EntityState are Enum instances (not raw strings) | PASS | ENT-002, ENT-003 |
| T-ENT-002 | kill() sets alive=False and state=EntityState.DEAD atomically | PASS | ENT-001 |
| T-ENT-003 | Dead entity position unchanged after step() | PASS | ENT-001 |
| T-PED-001 | XY speed ∈ [1.0, 1.8] m/s over 50 steps × 30 pedestrians | PASS | PED-001 |
| T-PED-002 | Z velocity = 0.0 every step (terrain lock) | PASS | PED-001, Req-7 |
| T-PED-003 | Z position = terrain elevation every step | PASS | Req-7 |
| T-PED-004 | is_eoi flag persists across 5 steps | PASS | PED-004 |
| T-PED-005 | Heading = atan2(vy,vx) after step | PASS | ENT-001 |
| T-MGR-001 | Duplicate entity_id raises ValueError | PASS | ENT-001 |
| T-MGR-002 | living() excludes dead entities | PASS | ENT-001 |
| T-MGR-003 | by_type() returns correct entity subset | PASS | ENT-002 |
| T-WLD-001 | in_bounds() returns True for interior position | PASS | ENV-001 |
| T-WLD-002 | in_bounds() returns False for exterior position | PASS | ENV-001 |
| T-WLD-003 | snap_to_terrain() sets Z = terrain elevation, preserves XY | PASS | Req-7, ENV-001 |
| T-WLD-004 | NFZ cylinder correctly identifies violations | PASS | ENV-005 |
| T-WLD-005 | AABB structure correctly identifies footprint containment | PASS | ENV-003 |
| T-BUS-001 | Subscriber receives published event of correct type | PASS | SIM-002 |
| T-BUS-002 | unsubscribe() prevents further delivery | PASS | SIM-002 |
| T-BUS-003 | Event published to wrong type not delivered | PASS | SIM-002 |
| T-ENG-001 | step_idx = floor(duration / dt) after run() | PASS | SIM-001 |
| T-ENG-002 | sim_time = step_idx × dt after run() | PASS | SIM-001 |
| T-ENG-003 | OOB entity killed within 5 steps (entity placed at X=99.95, speed_mps=1.8) | PASS | ENV-001 |
| T-ENG-004 | Determinism: identical seed produces identical position lists | PASS | SIM-003 |
| T-LOG-001 | JSONL step record written with correct step and entity ID | PASS | LOG-001 |
| T-LOG-002 | wall_dt_ms column present and correct | PASS | LOG-005 |
| T-LOG-003 | File handle closed after context manager exit | PASS | LOG-001 |
| T-LOG-004 | log_event() writes parseable JSONL record | PASS | LOG-002 |
| T-LOG-005 | Exception mid-run does not leak file handle | PASS | LOG-001 |

Final Result: 28 / 28 PASS. M1 Revised Baseline is cleared for M2 development.

## 16.4 Implementation Notes for Claude (Updated)

The following guidance supplements Appendix B from PRD v1.0:

- Never use @dataclass for classes that own mutable numpy arrays. Use explicit __init__ and document each field with a type annotation.

- All categorical fields (entity type, event type, FSM state, autopilot mode) MUST be Python Enum values — never raw strings. This is now a MUST requirement (ENT-002, NF-CE-002).

- Every new class requiring file I/O MUST implement the context manager protocol (__enter__ / __exit__). Use pathlib.Path, never str, for file paths.

- Speed normalisation in PedestrianEntity MUST use vector magnitude scaling and 2-D rotation — not additive Cartesian noise. Test with 50 entities × 500 steps to confirm envelope.

- snap_to_terrain() MUST be called by entity spawners before PedestrianEntity construction. Never pass a random Z coordinate as pedestrian altitude.

- The SimDefaults frozen dataclass is the ONLY place scalar constants may be defined. Any new tunable parameter introduced in M2+ MUST be added there first.

- Each milestone must be delivered with ≥ 80% test coverage measured by running the unittest suite. Tests must reference Req IDs in their docstrings.

- For GPU acceleration (NF-HA-001): introduce an xp alias (xp = cupy if cupy else numpy) in a dedicated gpu_utils.py module. All compute-intensive modules import xp, not numpy directly.

— END OF DOCUMENT —

UNCLASSIFIED // FOR OFFICIAL USE ONLY